Dredging Works at Indian Coast Guard Jetty, Porbandar Port

Capital Dredging | Gujarat Maritime Board | Porbandar, Gujarat

| CLIENT Gujarat Maritime Board (GMB) | LOCATION Porbandar Port, Gujarat, India | CONTRACT VALUE ₹13.88 Crore (excl. GST) |
| --- | --- | --- |
| SCOPE Capital dredging to 6.3m BCD | DREDGE QUANTITY 1,09,970 m³ (estimated) | DURATION 6 months (excl. monsoon) |
| Start Date – 25/11/2025 |  |  |

Project Overview

Porbandar Port, situated on the Arabian Sea coast of Gujarat, is an all-weather, direct-berthing commercial port serving merchant vessels, naval ships, and Indian Coast Guard (ICG) ships, alongside an active fisheries harbour. The Indian Coast Guard operates a round-the-clock station at the port, and the Gujarat Maritime Board (GMB) engaged our team to carry out capital dredging in front of the existing and proposed extension of the Coast Guard Jetty to ensure smooth, unhindered movement of ICG vessels.

The project required dredging the berth pocket to a design level of 6.3m BCD, executed within a live operational environment shared with cargo vessels, naval ships, and coast guard traffic - demanding a dredge spread capable of being relocated at short notice whenever shipping movements required it.

Location

Porbandar Port lies on the open Arabian Sea coast at approximately 21°38' N, 69°37' E. The dredging footprint covered the berth face of the existing and proposed Coast Guard Jetty, with all dredged material transported and disposed of at a designated offshore dumping ground in waters deeper than 20m below MSL — positioned well clear of the shoreline to prevent sediment drift back into the port or fisheries area.

| Port Coordinates 21°38' N, 69°37' E | Disposal Ground (Offshore) 21°36'5.00" N, 69°29'11.74" E |
| --- | --- |
| Water Body Arabian Sea (Open Sea, all-weather direct berthing port) | Tidal Range (MHHW–MLLW) +2.50m to +0.80m (Admiralty Chart No. 2054) |

Scope of Work

The work covered the full dredging cycle for the Coast Guard berth pocket:

| Dredging Location | Level to be Achieved | Est. Quantity |
| --- | --- | --- |
| In front of existing & proposed extension of the Coast Guard Jetty, Porbandar Port | 6.3m BCD (+300mm vertical / +500mm horizontal tolerance permitted) | 1,09,970 m³ |

- Mobilization of a suitable dredge spread (with ancillaries) capable of handling fine sand, clay/silt, pebbles, stray boulders, and hard strata encountered in the Porbandar seabed.

- Dredging of the berth pocket to 6.3m BCD, within permitted vertical (+300mm) and horizontal (+500mm) tolerances.

- Transport and disposal of dredged spoil at the identified offshore ground, beyond 20m MSL depth.

- Continuous hydrographic surveying - setting out, water-level registration, and pre-, interim- and post-dredge surveys - to certify quantities and levels achieved.

- A traffic-compatible work methodology allowing the dredge spread to be relocated at short notice to keep the channel clear for ICG, naval, and commercial shipping movements.

Methodology & Execution Approach

Equipment & Production Planning

The method statement submitted for this project detailed the dredging equipment deployed, its capacity and per-hour output, and the expected production rates for each unit of plant mobilized - enabling a realistic execution schedule to be built around the 6-month completion window (excluding monsoon).

Dredging Methodology

A detailed bar chart sequenced the deployment of equipment and the order of work across the jetty frontage, with allowances built in for weather downtime and site constraints. Because the dredge area sits directly in an active shipping channel, the spread was planned for rapid shifting at short notice, ensuring Coast Guard and other vessel movements were never obstructed during execution.

Disposal Plan

All dredged material was barged to the designated offshore disposal ground (21°36'5.00" N, 69°29'11.74" E), in water depths exceeding 20m below MSL, in line with the environmental safeguard of keeping disposed sediment from migrating back toward the shoreline.

Survey Methodology

Pre-dredge, interim, and post-dredge hydrographic surveys were carried out to verify achieved levels and compute dredged quantities (per the Simpson's Rule method), supported by continuous water-level registration and full setting-out of the work area with markers and beacons.

Key Challenges & How They Were Managed

- Live shipping channel: dredging had to proceed without disrupting Coast Guard, naval, and commercial vessel movements - addressed through a mobile, quickly-relocatable dredge spread.

- Variable seabed strata: the scope anticipated everything from fine sand to hard strata and rock mass, requiring a dredge spread specification flexible enough to handle mixed ground conditions.

- Environmentally sensitive disposal: spoil had to be placed far enough offshore and deep enough to prevent any impact on the shoreline or fisheries area, requiring precise navigation to the designated ground.

Project Facts

Awarding Authority: Gujarat Maritime Board — Office of the Executive Engineer (Mech.), PIU Porbandar

Estimated Contract Value: ₹13,87,82,140 (excluding GST)

Contract Period: 6 months (excluding monsoon) from date of work order

Design Dredge Level: 6.3m BCD

Dredged Quantity: 1,09,970 m³